Countersinking:

- What is the difference between countersinking and pre-drilling?

  - Countersinking uses a beveled entry to make the screw sit flush; as a result, the hole can then be filled with a wood plug appropriately

Every Beginning Woodworker Needs to Know This (Checking for Square):

- “Checking for square” just means checking all sides of a wooden board for squareness.

  - In a bigger sense, though, it also menas checking the squareness of a large project (e.g. a picture frame, cabinet, etc.)

- How do we do this? Let’s start w/ a combo or precision square. Tutorial (on a board):

  - 1. Choose a reference face (from which all else will be compared)

  - 2. Check it for flatness

    - It will be flat if you don’t see any light coming in beneath the blade (maybe even hold it up to light to check!)

  - 3. Look for squareness of edges:

  - 4. Look for squareness of the ends:

- Best way to fix an edge or face is a jointer

  - If you don’t have a jointer, you should look for S4S lumber, which is supposed to be all squared up

- Best way to fix the end of a board is by sawing it off

- Squaring up projects:

  - Note that you want to avoid paralellograms when you are supposed to be woodworking squares/rectangles!

    - Parallelograms can have compounding

  - To do this, you should first confirm that the 4 sides are appropriately equal

  - With a tape measure, you can also confirm that the diagonals are equal!

Types of Wood Clamps:

- Quick Grip Bar clamp (~150 lbs of clamping force)

  - Note that these clamps can be used for spreading in addition to clampimg, by simply re-attaching the non-fixed end

  - Disadvantage = free degreees of motion outside of the clamping plane

- Clutch Style Bar Clamp: